Review

- EIGRP

Distance-Vector Routing Protocol

- Neighbor Table
- Topology Table - Best route and backup route

- Routing Table - Best route 

*Auto-Summary
*Split-horizon

router eigrp AS

no auto-summary 
network 192.168.1.1 0.0.0.0 or 192.168.1.1 0.0.0.255

network 172.16.1.1 0.0.0.0

network 10.1.1.1 0.0.0.0

passive int fa0/1

Administrative Distance - If Multiple Routing protocol running Lowest First.

Static Route = 1

EIGRP = 90

OSPF =110

OSPF - 
Link-State Routing Protocol
Administrative Distance = 110

- Neighbor Table

- Topology Table - Database - FULL ROUTE

- Routing Table - Best route

EIGRP and OSPF are Classless 
Classful=not subnetted
Classless=subnetted

EIGRP
IP,IPX,AppleTalk

Enable Routing Protocol OSPF
router ospf process_id=1

net 192.168.1.2 0.0.0.0 area 0

net 172.16.1.2 0.0.0.0 area 1

net 10.1.1.2 0.0.0.0 area 2

int loopb 0

ip address 10.1.1.1 255.255.255.0

router eigrp 30

no auto-summary

network 10.1.12.1 0.0.0.0

net 10.1.1.1 0.0.0.0

passive-interface loopback 0