Switch Command

sh ip protocols
sh ip ospf neighbor

sh ip route ospf
sh ip ospf database
sh ip ospf interface

Classless Protocol is Subnetting

Classful does not support subnetting

10.4.4.1/32 [110/31] via 10.1.12.2, 01:05:06, FastEthernet0/0

Review

EIGRP 	IGRP		OSPF		Static Route
Administrative Distance	90		100		110			1

Routing Protocol	DistanceVector	 		- Link State - 		
			Route by Rumor			Full Route

Auto-Summary

Split-Horizon
			Hello Every 5 sec			Hello every 10 sec

Hold timer 15 sec			Dead Timer 50 sec
EIGRP to calculate best route they use Dual Algorithm 
OSPF uses SPF Algorithm

Unicast - One-to-one communication

Multicast - One-to-Many
Broadcast - One to All

EIGRP uses 224.0.0.10 multicast address to send hello

OSPF uses 224.0.0.5 and 6 to send hello
RIP - V1 uses classful
RIP - V2 uses classless
IGRP is classful 
EIGRP supports Classless

EIGRP 						OSPF

Topology keeps					Topology 
	Best route / Successor			Full Route
	Backup Route	/ Feasible Successor			Best Route 
								if best route is lost

OSPF has DR and BDR
Designated Router and Backup Designated Router
Based off the Segment they choose highest router ID to become designated router

Second highest becomes Backup Designated
When OSPF is enabled if you have a loopback the highest IP will be your Router IP
if no loopback then the router will use Physical Interface IP for Router ID.