HSRP
VRRP 
GLBP



HSRP Create IP and Group Number not recommended to use group 0
first create Virtual IP
When you setup a virutal IP you need to pick a unique IP

need to match IP and group number

int fa0/0

standby 1 ip 192.168.1.254

STANDBY is HSRP

STANDBY 10 track fa0/0 10

priority

preempt

With VRRP you can use one of the IP for Virtual IP --

VRRP lets you use actual IP address

R2(config)#track 1 interface serial 0/0 line-protocol ?

<cr>

The skew time is calculated as ((256 – VRRP priority) / 256)

(3 * 1) + ((256 – 100)/256)  = 3.609 seconds

HSRP			VRRP

CISCO			OPEN STANDARD
Timers 3/10		1/3SKEW

Preempt dis		Preempt enable

Can’t use Physical IP	CAN USE PHYSICAL IP

Track without Object	Track Object