Static route - 

Default route 0.0.0.0 0.0.0.0 10.0.0.1

ip route destinationIP destination subnet

Static routing 
	Static Route

Default Route
config(terminal) IP ROUTE

Dynamic Route
Distance Vector routing protocol (Auto Summarize) (NeighborTable-TopologyTable-RoutingTable)

EIGRP				(Split-Horizon)
	RIP - v2
Link state protocol	(Neighbor Table - Topology Table - Routing Table)

OSPF

EIGRP					OSPF (Open Shortest Path First)

DV					LS

Classless				Classless
IP/IPX/AppleTalk			IP

Topology Table			Topology Table(Database - FULL ROUTE)
(Best Route/Backup Route)		Hello 10 seconds
(Successor/Feasible Success)	40 second dead timer
Sends hello every 5 second		224.0.0.5

15 second hold timer			224.0.0.6

Wildcard Bit / Reverse Mask

xxx.xxx.xxx.xxx

Subtract from - 255.255.255.255

Example;   	  255.255.255.252
		- 255.255.255.255

0.0.0.3

Access List

- Standard Access List
		Only looks at Source information

- Extended Access List

Looks at source and destination and Layer 4 information/Transport

Implicit Deny/ any Deny

global config; access-list
(config-if)#ip access-group 1 in

(config)#access-list 100 permit tcp host 192.168.1.2 host 172.16.2.1 eq 23

(config-if)#ip access-group 100 in

<1-99>            IP standard access list

<100-199>         IP extended access list