Decimal 	128	64	32	16	8	4	2	1
Binary		0	0	0	0	0	0	0	0
Netmask 	128	192	224	240	248	252	254	255
Networks* 	2	4	8	16	32	64	128	256

IP Address Table

| Class | Leading bits | Start | End | Default Subnet Mask | CIDR notation |
| --- | --- | --- | --- | --- | --- |
| A | 0 | 0.0.0.0 | 127.255.255.255 | 255.0.0.0 | /8 |
| B | 10 | 128.0.0.0 | 191.255.255.255 | 255.255.0.0 | /16 |
| C | 110 | 192.0.0.0 | 223.255.255.255 | 255.255.255.0 | /24 |
| D | 1110 | 224.0.0.0 | 239.255.255.255 |  |  |
| E | 1111 | 240.0.0.0 | 255.255.255.255 |  |  |

IP is typically written in Dotted-decimal notation (DDN) 
 	  256
  	- Netmask
 	= Magic Number
Find Magic Number /CIDR Sum of Bits
Calculate number of networks and hosts

Networks: 2^x
*x = number of on bits (Borrowed Bits)
Hosts: 2^y -2
*y = number of off bits -2
2^16 = 65536
2^15 = 32768
2^14 = 16384
2^13 = 8192
2^12 = 4096
2^11 = 2048
2^10 = 1024
2^9   = 512
2^8   = 256
2^7   = 128
2^6   = 64

Cisco Switch Commands; 
en = Enable Privileged mode
config t = global config

Basic Switch Config Steps
config t) Hostname
config t) clock Timezone  PST -8
config t) clock Summer-time PDT recurring
Switch #) Clock set hh:mm:ss DD/Month YYYY

- Passwords
enable secret
config t# console line 0 
login
banner motd 

config t)# no vlan 2-1000
config t) vlan 1, 2, 3,
config t) int range fa 0/1 - 24

Configuring Interfaces / SwitchPort(s)

This shows interface configuration
sh run int fa0/1-x

Show run interface fastethernet 0/1

sh int fa0/1 sw

Setting Port Modes Access/Trunk/Dynamic
config t) int fa0/1
switchport mode access
switchport mode dynamic ? 
switchport trunk encapsulation dot1q
switchport mode trunk (DTP ”Negotiation”  Dynamic Trunking Protocol is enabled by default) 
disable DTP “nonegotiate”  
config if) switchport nonegotiate