Spanning-Tree

Common Spanning Tree (CST) "Not Cisco Supported” One instance
PVST - Per Vlan Spanning Tree - Default 
 	Block Time 20 seconds
 	Listening 15 seconds
 	Learning 15 seconds

RAPID-PVST (Best Practice) 
MST --- Multiple Spanning Tree

http://www.cisco.com/en/US/tech/tk389/tk621/technologies_white_paper09186a0080094cfc.shtml

Spanning-Tree Portfast
Spanning-Tree BPDUGuard Enable

Allow vlan on trunk ports

en

config t

hostname Sw1

clock timezone PST -8

clock summer-time PDT Recurr

VTP MODE TRANSPARENT

Spanning-Tree mode rapid-pvst

vlan 100,200,300

exit

int fa0/3

sw mode access

sw access vl 10

spanning-tree portfast

spanning-tree bpduguard enable

exit

default int range fa0/1-2

int range fa0/1-2

shut

sw trunk enc dot

sw mode trunk

channel-group 1 mode desir

no shut