Best Practice for PC to be in ACCESS PORT mode on switch.

Switch config

config t# vtp mode transparent

config t# spanning-tree mode rapit-pvst

config t# vlan 10,20,30

# sh run int fa0/0
#sh run int fa0/3

#sh int trunk

config-if# switchport trunk allowedSW1

(config-if)#sw trunk allowed vlan ?

WORD    VLAN IDs of the allowed VLANs when this port is in trunking mode

add     add VLANs to the current list

all     all VLANs

except  all VLANs except the following

none    no VLANs

remove  remove VLANs from the current list

SW1(config-if)#switchport trunk native ?

vlan  Set native VLAN when interface is in trunking mode

(config t)#default int fa0/0

#sh int status

VTP Mode Transparent or OFF is Best Practice

1 Root Bridge - Lower MAC 
2 root Port lower int number

3 DP
4Root Block

CTRL 6 will stop command on Cisco router

sh spanning-tree

Etherchannel - Creates Logical Interface called Port Channel

Turn on
Or use negotiation protocol

PAgP Port Aggregation Protocol
Desirable 
Auto

LACP Link Aggregation Control Protocol
Active
Passive

best way way to configure is shutdown the interface 
Interface configuration has to be identical

SW1(config-if-range)#channel-group 3 mode ?

active     Enable LACP unconditionally

auto       Enable PAgP only if a PAgP device is detected

desirable  Enable PAgP unconditionally

on         Enable Etherchannel only

passive    Enable LACP only if a LACP device is detected

SW1#sh etherchannel summary

SW1#sh int status

sh etherchannel summary

sh run int po X

sh ip in br

en

config t

hostname Sw1

clock timezone PST -8

clock summer-time PDT Recurr

VTP MODE TRANSPARENT

Spanning-Tree mode rapid-pvst

vlan 10,20,30,40,50

exit

int fa0/3

sw mode access

sw access vl 10

spanning-tree portfast

spanning-tree bpduguard enable

exit

default int range fa0/1-2

int range fa0/1-2

shut

sw trunk enc dot

sw mode trunk

channel-goup 1 mode on

no shut