# CCNA Cisco commands

Decimal 		128	64	32	16	8	4	2	1

Binary  		0	0	0	0	0	0	0	0

Subnet Masks		128	192	224	240	248	252	254	255

Networks		2	4	8	16	32	64	128	256

-----------------------------------------

MODE

-----------------------------------------

Switch>			User Mode

Switch#			Privileged mode

Switch(config)#		Global configuration mode

Switch(config-if)#	Interface mode

-----------------------------------------

Basic Switch Config

-----------------------------------------

Switch> enable

Switch# configure terminal

Switch(config)#

Switch(config)#hostname Cisco

Cisco(config)#

-----------------------------------------
Mode
-----------------------------------------

switch> 		User mode
switch#		Privileged mode

switch(config)#	global config
switch(config-if)# 	Interface config mode

switch(config-line)	configure console

-----------------------------------------

basic switch config

-----------------------------------------

switch>enable  		<!- enter privileged mode -!>
switch# configure terminal	<!- enter global configuration mode -!>
switch(config)#		<!- global configuration mode -!>
switch(config)#hostname NAME	<!- hostname -!>

switch# copy running-config startup-config  <!-- Save current configuration --!>
switch# show running-config  		<!-- running config is stored in ram --!>
switch# show startup-config 			<!-- Startup config is stored in NVRAM --!>

-----------------------------------------
password
-----------------------------------------
switch(config)#enable password WORD 		<!-- Plain text password --!>
switch(config)#enable secret WORD		<!-- MD5 password --!>
switch(config)# service password-encryption	<!-- encrypts clear text passwords --!>
switch(config)# line console 0 			<!-- Console configuration --!>
switch(config-line) password WORD
switch(config-line) login 				<!-- Enables console password --!>

-----------------------------------------
setup clock
-----------------------------------------

*configure timezone before setting clock(Global config)
switch(config)# clock timezone pst -8
switch(config)# clock summer-time PDT recurring
switch# clock set hh:mm:ss 1-31 month YYYY  
switch# show clock			<!-- Show clock date/time --!>

-----------------------------------------
Banner
-----------------------------------------
switch(config)#banner motd #
======================================
===		Authorized Personnel Only!	      ===
======================================
#

-----------------------------------------
Clear switch configuration
-----------------------------------------
switch# delete flash:vlan.dat
switch# clear startup-config
switch# reload

-----------------------------------------
vlan configuration
-----------------------------------------

switch(config)#vlan 1-4096 	<!-- Create

-----------------------------------------
Assign Access Mode
-----------------------------------------

switch(config)#interface fastethernet 0/1
switch(config-if)#switchport mode access

-----------------------------------------
Assign Trunk Mode
-----------------------------------------

switch(config)# interface fastethernet 0/1
switch(config-if)# switchport trunk encapsulation dot1q

switch(config)#switchport mode trunk

switch(config-if)#sw trunk allowed vlan

-----------------------------------------
Switch Protocols
-----------------------------------------

Trunking Protocol (Language) 
	ISL(Inter-Switch-Language) 
		-Adds 26 Bytes header and 4 Bytes trailer
	802.1q(dot1q-IEEE Standard)
		-Adds 4 Bytes tag

DTP (Dynamic Trunking Protocol) - Negotiates Trunking
Dynamic Desirable - on (can’t turn off)
Dynamic Auto on (Can’t turn off)
Trunk - on (can turn off) 
	-switchport nonegotiate

-----------------------------------------
DTP (Dynamic Trunking Protocol)
-----------------------------------------

Switch(config)#interface fastethernet 0/1

Switch(config-if)#switchport mode dynamic desirable 	→ active

Switch(config)#interface fastethernet 0/1

Switch(config-if)#switchport mode dynamic auto

Switch(config-if)#switchport voice vlan 10

Switch(config-if)#switchport allow vlan 10,20

--------------------------------------------------

Best Practice for Access Port

--------------------------------------------------

Switch(config)#interface fastethernet 0/1

Switch(config-if)#switchport mode access

Switch(config-if)#switchport access vlan 10

Switch(config-if)#no cdp enable

Switch(config-if)#spanning-tree portfast

Switch(config-if)#spanning-tree bpduguard enable

--------------------------------------------------

Allow vlans and assign native vlan on a trunk

--------------------------------------------------

Switch(config)#interface 0/1

Switch(config-if)#switchport mode trunk

Switch(config-if)#switchport trunk native vlan 10

Switch(config-if)#switchport trunk allowed vlan 10,20

--------------------------------------------------

Setting IP Addresses on a VLAN

--------------------------------------------------

Switch(config)#interface vlan 10

Switch(config-if)#ip address 192.168.1.5 255.255.255.0

Switch(config-if)#exit

--------------------------------------------------

Switch Protocols

--------------------------------------------------

Trunking Protocol (Language)

ISL(Inter-Switch Link - Cisco Proprietary)

802.1q(DOT1Q-IEEE Standard)

DTP - (Dynamic Trunking Protocol) - Negotiates Trunking

Mode:    Dynamic Desirable

Dynamic Auto

VTP - (VLAN Trunking Protocol) - Replicates Vlans

STP - (Spanning Tree Protocol) - Prevents Switch Loops

CST

PVST

Rapid-PVST

MST

--------------------------------------------------

VTP (VLAN Trunking Protocol)

--------------------------------------------------

Switch(config)#vtp mode server

Switch(config)#vtp mode client

Switch(config)#vtp mode transparent (best practice)

Switch(config)#vtp domain CCNA

Switch(config)#vtp password 123

switch#show vtp status --> Shows general info about VTP config

by default all switches are in server mode

--------------------------------------------------

STP Election process

--------------------------------------------------

1. Root Bridge

*Lowest BID

2. Root Port

*fastest to root bridge

3. Designated Port

*All port in root bridge is DP

*One DP Per segment/link

4. Block Port

*Rest

--------------------------------------------------

Configuring the root switch

--------------------------------------------------

switch(config)#spanning-tree vlan 10 priority 0

--------------------------------------------------

Configuring Rapid STP

--------------------------------------------------

switch(config)#spanning-tree mode rapid-pvst

--------------------------------------------------

Switch Port Security

--------------------------------------------------

Switch(config)#interface fastethernet 0/1

Switch(config-if)#switchport port-security

Switch(config-if)#switchport port-security maximum 1

Switch(config-if)#switchport port-security violation shutdown

Switch(config-if)#switchport port-security mac-address sticky

switch#clear port-security sticky interface fastethernet 0/1

--------------------------------------------------

Switch Protocols

--------------------------------------------------